# Stacking gel solution (3.75)

# for SDS-PAGE (18.5 cm  20 cm gels)

Prepare: the comb, the flask with rubber stopper and box with ice. Keep a flask on ice during the preparation of solution.

For the gels of 1 mm thickness:

|  | 20 ml (2 gels) | 10 ml (1 gel) |
| --- | --- | --- |
| Acrylamide/Bis-acrylamide (stock solution 30, w/v; crosslinker ratio 29:1; BIO-RAD, Cat.No: 161-0156) | 2.5 ml | 1.25 ml |
| TRIS-HCl 2M, pH 6.8 (0.149 M final) | 1.49 ml | 745 microl |
| Distilled H2O | 15.77 ml | 7.885 ml |
| SDS 10 (0.12 final) | 240 microl | 120 microl |

Degas under vacuum for 15 min (do not forget to keep the flask with solution on ice). Then add:

| TEMED (N,N,N,N’-Tetra-methylethylenediamine; BIO-RAD, Cat.No. 161-0800) | 72 microl | 36 microl |
| --- | --- | --- |
| Ammonium persulfate 10, w/v (0.06 final) | 120 microl | 60 microl |

Fill the gel cassette with stacking gel solution and insert a comb (avoid air bubbles), and allow for polymerization (at least 30 min). When a gel is polymerized, cover the top of the gel with Parafilm and put it into a cold room overnight for better polymerization.

A comb is used to generate wells for sample loading. Depending on the application, analytical (10-16 wells) or preparative scale (1-2 wells) combs can be used. The effective stacking gel height is 1.5 cm.

The gel may be stored for up to two weeks (at 4C, fully immersed in ‘running buffer’). Prolonged storage could result in dissolution of the sharp pH boundary between the lower pH (8.8) and upper pH (6.8) buffers.

Solutions: Tris T-1378 ‘Sigma’ 24.22g/100ml H2O, pH 6.8 with HCl

2002 Julius